# Comprehensive Audio Processing Notes (Spectrogram, STFT, MFCC)

# 1. Spectrogram

A spectrogram is a 2D visualization of audio signals showing how frequency content evolves over time.

Axes:
- X-axis: Time (frames)
- Y-axis: Frequency (Hz)
- Color: Magnitude/energy of frequency components

It is generated using STFT and is widely used in speech recognition, music analysis, and audio classification.

# 2. STFT (Short-Time Fourier Transform)

STFT divides the signal into small overlapping windows and applies FFT to each window.

Steps:
1. Split audio into frames
2. Apply window function (e.g., Hamming)
3. Compute FFT for each frame

Purpose: Capture both time and frequency information simultaneously.

# 3. Sample Rate

Sample rate is the number of samples taken per second from a continuous signal.

Formula:
Sample Rate = samples per second (Hz)

Higher sample rate → better time resolution and quality
Lower sample rate → less data, lower quality

Nyquist Theorem: Max frequency captured = sample_rate / 2

# 4. Bit Depth

Bit depth defines how precisely amplitude is stored.

Higher bit depth:
- More dynamic range
- Less quantization noise

Dynamic Range ≈ 6 × bit depth (in dB)
Example: 16-bit ≈ 96 dB

# 5. Audio Channels

Channels represent independent audio streams.

Mono: Single channel
Stereo: Left + Right
Surround: Multiple channels for spatial audio

Used to create directional and immersive sound experiences.

# 6. Window Size & Hop Length

Window size defines how much signal is processed at once.
Hop length defines how much we shift forward.

Overlap = Window size - Hop length

Trade-off:
- Larger window → better frequency resolution
- Smaller window → better time resolution

# 7. n_fft

n_fft is the number of samples used to compute FFT.

Frequency bins = (n_fft / 2) + 1

Higher n_fft:
- Better frequency resolution
- More computation

Typically chosen as power of 2 (e.g., 512, 1024, 2048)

# 8. MFCC

MFCCs represent audio in a compressed form based on human hearing.

Steps:
1. Apply STFT
2. Convert to Mel scale
3. Take log
4. Apply DCT

Captures timbral/spectral characteristics of audio.

# 9. MFCC Shape

MFCC output shape = (n_mfcc, number_of_frames)

Example: (13, 123)
- 13 coefficients per frame
- 123 time frames

Transpose needed for ML: (123, 13)

# 10. Delta & Delta-Delta

Delta features capture temporal changes.

Delta = first derivative (velocity)
Delta-Delta = second derivative (acceleration)

Final feature vector often = 39 dimensions

# 11. Overlapping

Overlapping ensures continuity between frames.

Benefits:
- Reduces boundary information loss
- Produces smoother features
- Improves ML model performance

# 12. Duration from MFCC

Duration can be approximated using:

duration = (num_frames × hop_length) / sample_rate

Exact duration should be computed from raw signal length.

# 13. Typical Values

Speech Processing Standards:
- Window size: 20–40 ms
- Hop length: 10–20 ms
- MFCC: 13
- n_fft: 512–2048

These values balance time and frequency resolution.